ANUDEM

------

A program which calcuates values on a regular grid of a discretized smooth

surface fitted to large numbers of irregularly spaced elevation data

points, contour line data and stream line data.  The program imposes a

global drainage condition which automatically removes sinks where possible.

The program is written in standard FORTRAN 77 with one exception (see

Appendix 2).

Author: M.F. Hutchinson, Centre for Resource and Environmental Studies,

Australian National University, G.P.O. Box 4, Canberra ACT 2601.

Last revision to this document: 11 March 1991.

Introduction.

------------

The program has been designed to produce accurate digital elevation models

with sensible drainage properties from comparatively small, but well

chosen, elevation and stream line data sets.  It normally achieves this by

imposing two critical, but optional, assumptions on the input elevation

data:-

1. That the elevation data points include ALL significant local maxima.

2. That the input data include ALL significant local minima (sink

points).

The algorithm implemented by the program interpolates the elevation data

onto a regular grid by minimizing a suitably weak roughness penalty on the

fitted grid values and by simultaneously imposing constraints which take

into account these two assumptions.

The first assumption, which is optional, is incorporated simply by

permitting local maxima to lie only at grid points to which data points

have been allocated.  This removes the spurious peaks away from data points

often generated by general purpose interpolation techniques.  The first

assumption may be dropped when input elevation data consists principally of

detailed contour lines since these data do not normally include local

maxima.  The program then interpolates heights of local maxima on the basis

of the surrounding contour height information.

The second assumption is incorporated by imposing a global drainage

condition on the fitted grid values which automatically removes spurious

sinks or pits. This is the principal innovation of the program.  It

eliminates one of the main weaknesses of elevation grids produced by

general purpose interpolation techniques which has limited their usefulness

in hydrologic applications, particularly those which rely on the automatic

calculation of catchment areas.

Moreover, the imposed global drainage condition has been found in pratice

to be a powerful condition which can significantly increase the accuracy,

especially in terms of their drainage properties, of digital elevation

models interpolated from sparse sets of surface specific data.  The size of

such data sets can be at least an order of magnitude smaller than the

number of points normally required to adequately describe elevation using

digitized contours. This minimizes the expense of obtaining reliable

digital elevation models in terms of the capture, correction and storage of

primary elevation data.  The global drainage condition also virtually

eliminates the need for detailed manual editing of interpolated elevation

grids to remove spurious drainage features.

The second of the two assumptions above is not as critical as the first,

since the program acts conservatively when attempting to remove sinks and

does not impose drainage conditions which would plainly contradict the

elevation data.  A consequence of this is that errors in both elevation and

position of input elevation data can often be indicated by sinks in the

final fitted grid, especially when the input data includes at least the

principal stream line networks.  This is highly useful when processing very

large data sets, and the program can optionally write out diagnostic

information for each sink to assist in the correction of data errors.  The

number of such sinks is usually quite small.  The conservative nature of

the program imposed drainage conditions also makes the program quite robust

to moderate errors in the positions of input stream line data and capable

of producing generalized (coarse resolution) elevation models with

appropriately generalized drainage properties.

Drainage enforcement algorithm.

------------------------------

The global drainage condition is imposed by attempting to remove all sink

points which have not been identified as such in input sink data files.

The essence of the drainage enforcement algorithm is to find for each sink

point the lowest adjacent saddle point which leads to a lower data point,

sink or edge, and then, provided an elevation data point has not been

allocated to the saddle, enforcing a descending chain condition from the

sink via the intervening saddle to the lower data point, sink or edge.

This action is in fact modified by the systematic application of three user

supplied tolerances. These tolerances allow the user to adjust the strength

of drainage enforcement in relation to both the accuracy and density of the

input elevation data.

The detailed action of these tolerances has undergone considerable

development and testing with data sets of varying densities and accuracies

at a variety of scales.  The aim has been to achieve the strongest possible

drainage enforcement without making serious errors in the placement of

drainage lines, particularly when the input data is limited in terms of

accuracy or density. The action of the tolerances naturally becomes less

critical as the accuracy and density of the input data improves.  When the

tolerances have been set appropriately, the sink points not cleared by the

program are normally those associated either with significant elevation

errors in input data or with areas where the input data are not of

sufficient density to reliably resolve the drainage characteristics of the

fitted grid.

The first user supplied tolerance should principally reflect the elevation

accuracy of the input data points but can also reflect the density of the

input elevation data.  Elevation differences between data points not

exceeding this tolerance are judged to be insignificant with respect to

drainage.  Thus data points which block drainage by no more than this

tolerance are removed.  When data points are not sufficiently dense to

accurately resolve drainage, this tolerance may be increased somewhat and

will yield a slightly generalized drainage pattern at the expense of

fidelity to the elevation data.  This is especially useful when working at

smaller scales (less than say 1:100,000). The first tolerance is also used

when searching for possible clearances for sinks, to slightly favour those

saddle points which are not associated with data points over saddle points

which are associated with data points.

The second tolerance limits the height above each data point sink of data

point saddles which may be considered as possible exits from the sink.

This can remove from consideration certain data point saddles, even though

they are the lowest saddle associated with particular sinks, in order to

allow drainage clearance via a higher non-data point saddle.  This

tolerance normally has a significant effect only when data points are

relatively sparse.  Setting the second tolerance to its minimum allowable

value of twice the first tolerance is applicable when data point density is

only just sufficient to accurately resolve drainage characteristics.

Setting the second tolerance to a larger value ensures that the program

acts conservatively when attempting to remove sinks and consequently more

sinks may remain in the final grid.  This is appropriate when data point

density is not sufficient to completely resolve the drainage

characteristics of the fitted grid.  The user may wish to remove remaining

sinks by adding more data points or streamlines in critical areas in this

case.

Drainage enforcement can also be obtained by incorporating stream line

data. This is useful when more accurate placement of streams is required

than can be calculated automatically by the program.  It can also be used

to remove sinks which would not otherwise be removed by the automatic

drainage enforcement algorithm.  This is in fact the recommended way to

correct drainage anomalies in elevation grids calculated by the program.

All elevation data points which conflict with strict descent down each

stream line are removed, with all conflicts which exceed the third user

specified elevation tolerance being written to output for possible

correction of errors.  Side conditions are also set for each stream line.

These ensure that the stream line acts as a breakline for the interpolation

conditions and simultaneously ensure that each stream line lies at the

bottom of its accompanying valley.  Side conditions are not set for data

points beside streams whose elevations are more than the third tolerance

below the height of the stream.  These conflicts are written to output for

possible correction of errors.  The third elevation tolerance is also used

to prevent automatic clearances of sinks via saddles whose elevation exceed

the height of the sink by more than twice the value of the third tolerance.

Such situations are normally caused by significant elevation errors but

can also arise when data points are very sparse.

Outline of Program Structure.

----------------------------

The program can process up to MAXFIL different input data files where

MAXFIL is a program supplied parameter (currently 80).  Each data file

may be one of five types:-

1. Point elevation data

2. Sink point data

3. Stream line data

4. Polygonal line data

5. Contour line data

Detailed specifications of each type of data file are given below.  The

program first reads input data points from each input data file, windows

the data to the user specified map limits and then generalizes the data to

the user specified grid resolution.  Point data are generalized by

accepting at most MAXW data points per grid cell (currently MAXW=4) and

discarding any remaining points.  Line data are generalized by accepting at

most one line data point per grid cell, and, in the case of stream line

data, removing unnecessary kinks. The selected data points are written to

unformatted scratch files, one file for each of the five types of data, for

quick retrieval during program execution.

The program then employs a simple multi-grid method which calculates grids

at successively finer resolutions, starting from an initial coarse grid,

until the final, user specified grid resolution and simultaneously imposes

sensible drainage conditions to remove sinks where possible according to

the algorithm described above.  For each grid resolution, the accepted data

points are read from the five scratch files and values at grid points not

occupied by data points are calculated by Gauss-Seidel iteration with

overrelaxation (SOR method) subject to an appropriate roughness penalty and

ordered chain constraints.  The ordered chain constraints are obtained from

user supplied stream line data, sink point data and contour line data and

by automatic drainage enforcement as calculated by the program.  Iteration

terminates when the user specified maximum number of iterations (normally

25) has been reached. Starting values for the first coarse grid resolution

are calculated from a least squares plane fit to the data points. Starting

values for each successive finer grid are linearly interpolated from the

preceding coarser grid.

On completion of the multi-grid iterations, the program calculates all sink

points remaining in the fitted grid and optionally writes a detailed

summary to output.  These sink points are also optionally written to a user

specified file for plotting.  The program optionally calculates the true

root mean square residual from the fitted grid of the data points in their

original positions, using bilinear interpolation across the grid cells.

Stream line information, as incorporated onto the grid, is also optionally

written to a file in a format suitable for plotting.

Specifications of input data files.

----------------------------------

1. Elevation data points.

Each file contains ordered X,Y,Z triples which denote the position and

elevation of each data point, with  at most one data point per record.  For

each grid resolution, these data points are allocated to the nearest grid

point.  If more than one data point is allocated to the same grid point,

then the average value is taken, with no more than MAXW data points being

considered for each grid point.  MAXW is a program supplied parameter

(currently 4). There is no limit on the number of input elevation data

points read from the user supplied data files.

2. Sink data points.

Each file contains ordered X,Y,Z triples which denote the position and

elevation of nominated sink points.  The program will not attempt to remove

sinks at such points.  The height of a sink point can be left unspecified

by setting the Z value to a value lying outside the user specified height

limits (e.g. -999.0).

3. Streamline data points.

Each file contains strings of X,Y coordinate pairs, in order of DESCENDING

elevation, each string being preceded by a header which contains, in order,

the number of coordinate pairs in the following string, the height of the

first coordinate pair and the height of the last coordinate pair.  These

heights can be left unspecified, in a similar manner to sink point

elevations, by setting them to values lying outside the user specified

height limits.  However, the coordinate pairs in each streamline MUST be

ordered from highest point to the lowest point, since the values in the

grid are constrained to DESCEND down each streamline.  Streamline data

take priority over point elevation and contour line data.  Thus elevation

data points which conflict with descent down each streamline are ignored.

4. Polygonal data points.

Each file contains strings of X,Y coordinate pairs, each string being

preceded by a header which contains the number of coordinate pairs in the

following string.  Each string or consecutive group of strings must close

within a program calculated tolerance to form a closed polygon.  Polygons

with an area less than a small fraction of the area of one grid cell are

ignored.  Points in the final output grid which lie outside the polygons

specified by these line strings are set to a program determined special

value which is less than the user specified minimum height limit.  This

special value is written to output.  If no polygon strings are read then no

grid points are set to special values.

5. Contour lines.

Each file contains strings of X,Y coordinate pairs, each string being

preceded by a header which contains the number of coordinate pairs and the

height of the contour line.  The heights of the points making up the

contour are incorporated into the grid.

References

----------

Hutchinson,M.F. (1989). A new method for gridding elevation and

stream line data with automatic removal of pits. J. Hydrol. 106,

211-232.

Hutchinson,M.F. (1988). Calculation of hydrologically sound digital

elevation models. Proc. Third Inter. Symp. Spatial Data Handling,

August 17-19, Sydney. Inter. Geog. Union, Columbus, Ohio.

Hutchinson,M.F. and Dowling,T.I.(1991). A continental hydrological

assessment of a new grid-based digital elevation model of

Australia. Hydrological Processes 5, 45-58.

APPENDIX 1

User Directives.

---------------

User directive    Type              Description

------------------------------------------------------------------------

Drainage option   0,1,2 or 3        Level of drainage enforcement:

0 - no drainage enforcement;

1 - drainage enforcement where

possible (normal option);

2 - drainage enforcement with sink

diagnostics;

3 - drainage enforcement with full

diagnostics (not normally used).

Contour data      0 or 1            0 - data consists mainly of spot

option                                   heights;

1 - data consists mainly of contours.

Maximum ridge     Non-negative      Required only if contour data option

length            real (optional)   is 1.  Inferred ridge lines are only

enforced if their length is less than

the maximum ridge length, e.g. 500m.

No ridges are set if the maximum ridge

length is zero.  All ridges are set if

the maximum ridge length is set to a

very large value.

Root mean square  Non-negative      For elevation data consisting mainly

residual          real              of scattered spot heights, set to zero.

For elevation data consisting mainly

of contours, set to 0.5-1% of the

contour interval.

e.g. set initially to 0.05 if the

contour interval is 10.

Increase the residual if contours

calculated from the fitted grid at the

same heights as the data contours show

small crenulations (i.e. are crinkly).

Maximum option    0 or 1            If 0 then local maxima in the fitted

grid are not constrained to lie at data

points.  If 1 then local maxima in the

fitted grid are constrained to lie at

data points.  Normally 0 for both

contour line data and point data.  Set

to 1 if spurious maxima are generated

using 0 and the data consists of well

chosen point data which include all

significant local maxima.

Roughness penalty Non-negative      0.0 for minimum curvature (for contour

trade-off         real                  data);

0.5 for mixture of minimum curvature

and minimum potential (appropriate

for spot heights).

Larger values not normally recommended.

Elevation         Three non-        First tolerance reflects accuracy of

tolerances        negative reals    elevation data.  Second tolerance must

exceed the first tolerance.  Setting

the second tolerance to be twice the

first tolerance is a reasonable

compromise between strength of drainage

enforcement and elevation accuracy.

Set the second tolerance to a higher

value for poor quality or sparse data

or to ensure more conservative drainage

enforcement. When contour data is used

set the second tolerance equal to the

data contour interval.

Side conditions of streams are only set

if they do not conflict with the data

by more than the third tolerance and

saddles which exceed adjacent sink

heights by more than twice the third

tolerance are not cleared by the

drainage enforcement algorithm. See

notes on the drainage enforcement

algorithm above.

Typical values for adequate data at a scale

of 1:100,000 are:-

5.0  20.0  100.0.

Typical values for less dense data at a

scale of 1:500,000 are:-

10.0  20.0  150.0.

Typical values for contour data with a

contour spacing of 10 are:-

2.5  10.0   50.0.

Maximum number    Positive integer  Normally 30 is adequate for most

of iterations                       applications.  Set to a larger value

(say 35 or 40) if more sinks may

be cleared or more streams and ridges

may be set when using contour data.

Iteration ceases for each grid

resolution when the maximum number of

iterations has been reached.

Height limits     Two reals         Lower and upper height limits

respectively.  Data points outside

these limits are ignored, and the

fitted grid values are constrained to

lie between these two limits.

Residual file     40 characters     Name of file where all residuals larger

name                                than the specified minimum residual are

written.  If this is left blank then no

residuals are calculated.

Minimum residual  Non-neg real      Include only if the residual file name

(optional)        is not blank.  All data points whose

residual exceeds the minimum residual

are calculated and written to the  user

specified residual file.

The true root mean square residual from

the final grid of all data points, in

their true positions, which lie within

the user specified limits (and polygon

limits), is also calculated and written

to output.

X (longitude)     Two reals         Lower and upper X coordinate limits

limits                             respectively.  Data points outside

these limits are ignored.

Y (latitude)      Two reals         Lower and upper Y coordinate limits

limits                             respectively.  Data points outside

these limits are ignored.

Grid spacing      Non-neg real      The resolution of the final grid.

The spacing should divide the X and

Y ranges an integral number of times.

The number of grid points in the X

direction is

(upper X - lower X)/spacing  +  1

and the number of grid points in the

Y direction is

(upper Y - lower Y)/spacing  +  1.

Number of data    1-80              Number of data files, for each of which

files                               a name, a type,and formats have to be

supplied (see following options).

Data file name    40 characters     Name of input data file.

Type of data      1-5               1 - elevation data file

file                                2 - sink data file

3 - streamline data file

4 - polygon data file

5 - contour line data file

Data file header  40 characters     Header format - required only for data

format                              file types 3,4 and 5:-

3 - for reading no. of coordinate

pairs, height of first point, and

height of last point,

e.g. (10X,I10,2F10.3)

4 - for reading no. of coordinate pairs

only, e.g. (10X,I10)

5 - for reading no. of coordinate

pairs and height of contour line,

e.g. (10X,I10,F10.3)

Data file format  40 characters     For types 1 and 2 - X,Y,Z format, at

most one point per record,

e.g. (2F10.5,F10.3)

For types 3,4 and 5 - X,Y format, more

than one point per record allowed,

e.g. (8F10.5)

Output grid       40 characters     Name of output grid file.  The grid is

file name                           written row by row from minimum Y to

maximum Y.

Mode of output    0,1 or 2          0 - real grid values by rows;

grid                                1 - real X,Y,Z format;

2 - USGS (ARC/INFO) format.

Output grid       40 characters     For mode 0 - row format of output grid

format                                e.g. (10F8.2)

For mode 1 - X,Y,Z format

e.g. (2F10.5,F8.2)

Blank for unformatted ouput, in either

mode 0 or mode 1.

Not required for mode 2.

Output sink file  File name         If non-blank, remaining sink points in

the final grid are written to this file

for later plotting.

Streamline        File name         If non-blank, stream lines and ordered

diagnostic file                     chain conditions, as incorporated in

the final grid, are written to this

file for later plotting.

---------------------------------------------------------------------------

APPENDIX 2

Non-standard features.

---------------------

There is currently one non-standard features in the program,

which can be easily removed if necessary:

1. Messages accompanying terminal errors have been incorporated by

providing character strings with FORTRAN STOP statements.  This

non-standard extension is supported by many compilers (e.g. VMS, NOS,

UNIX).  The character strings may be replaced by integers when this

extension is not supported.